Sec. 122-625. - Approval of solar systems located in regulated historic districts.

Approval of solar systems located in regulated historic districts.

(1)  Design review required. Solar systems located in regulated historic districts shall obtain approval from the historic district commission, where applicable, before seeking any other required approvals.

(2)  Procedures for design review shall be in accordance with chapters 64 and 122. In addition to the applicable standards and regulations found therein, the following design standards apply for solar systems:

a.  Solar systems shall be installed in a manner which does not damage or obscure character-defining features of an historic resource. Solar panels should be located so as not to change an historic roofline or obscure the relationship of an historic roof to character-defining features such as dormers and chimneys.

b.  Pole-mounted solar systems shall be installed in locations that are not clearly visible from public streets.

c.  Roof-mounted solar systems that are not building integrated systems.

1.  Solar systems on pitched roofs:

a.  Systems may only be attached to the side or peak of a roof that is clearly visible from public streets when such systems could not otherwise be located on sides and peaks of the roof that are not clearly visible from public streets.

b.  Systems shall be installed below the ridgeline of a pitched roof.

c.  Systems shall be mounted parallel to the plane of a pitched roof and have a low profile.

2.  Solar systems on flat roofs:

a.  Systems shall not be attached to parapet walls that are clearly visible from public streets or any extension of a front wall of a building that is clearly visible from public streets.

b.  Systems shall be set back from the edge of a flat roof, so as not to be visible from a public street.

d.  Building integrated solar systems. The historic district commission, where applicable, shall review and approve the use of building integrated solar systems.

e.  The historic district commission, where applicable, may require that solar systems, excluding solar collector surfaces, be painted so as to be architecturally compatible with the building on which it is located or to blend in with the surrounding area.

(Ord. No. 2010-9, 6-21-10; Ord. No. 2012-8, § 1, 6-18-12)

Sec. 122-626. - Specific standards for solar systems.

(a)  Solar systems are a permitted use subject to the following standards:

(1)  Height. Solar systems shall meet the following height standards:

a.  Height of solar systems mounted on poles shall not exceed height requirements for principal uses as specified in article XIV, schedule of regulations.

b.  Height of solar systems that are roof-mounted and not within regulated historic districts may exceed the height of the tallest part of the roof to which it is attached by no more than ten feet.

c.  Applicants must obtain special use approval from the planning commission to exceed height restrictions

(2)  Setback requirements. Solar systems shall meet the following setback requirements:

a.  General rules for location on lot. Solar systems, with the exception of building integrated solar systems, may not encroach into setback requirements for accessory uses for the zoning district in which they are to be located; nor shall they be located between the front lot line and the front wall of dwellings.

b.  Pole-mounted solar systems. Solar systems shall be set back a distance equal to the height of the pole or solar system, whichever is greater, from all adjacent occupied structures.

c.  Roof-mounted solar systems.

1.  Location on roof. The collector surface and mounting devices shall not extend beyond the exterior perimeter of the building on which it is mounted.

2.  Exterior piping for solar hot water systems shall be allowed to extend beyond the perimeter of the side or rear of a building and shall be painted to blend in with adjacent surfaces.

(3)  Number.

a.  Pole-mounted. Solar systems may consist of two pole-mounted systems.

b.  Roof-mounted. Unlimited.

c.  Building integrated. Unlimited.

d.  Wall-mounted. Shall not cover more than 50 percent of the wall to which it is attached.

(4)  Installation. Solar systems may be mounted on poles, roof-mounted, or wall-mounted.

(5)  Guy wires. Poles requiring guy wires are not permitted in R-1, single-family residential and R-2 single and two-family residential districts.

(6)  Size of solar systems.

a.  Ground-mounted. Limited to lot coverage requirements for accessory structures.

b.  Roof-mounted. May not exceed the square footage of the roof to which they are attached.

c.  Building integrated. No limit to the size of building integrated systems.

d.  Wall-mounted. Shall not cover more than 50 percent of the wall to which it is attached.

(7)  Minimum lot size. None.

(b)  Where a solar system is the principle use of the property, it is only permitted after special use approval in the M-1, light industrial and M-2, general industrial districts subject to the following standards:

(1)  Height. Subject to special approval.

(2)  Square footage of systems. Subject to special approval.

(3)  Setback requirements. Solar systems shall meet the following setback requirements:

a.  Solar systems may not encroach into setback requirements for principal uses for the zoning district in which they are to be located.

b.  Solar systems shall be set back a distance equal to 100 percent of the system height from all property boundaries, public rights-of-way, utility lines, or the Saginaw River.

(4)  Number. Subject to special approval.

(5)  Installation. Solar systems may be ground-mounted, roof-mounted or on poles.

(6)  Minimum lot size. Subject to special approval.

(7)  Decommissioning. The applicant shall submit a decommissioning plan that is in recordable form to be recorded with the county register of deeds. The plan shall include the anticipated life of the project, the estimated decommissioning costs net of salvage value in current dollars, the method of ensuring that funds shall be available for decommissioning and restoration and the anticipated manner in which the project shall be decommissioned and the site restored.

(8)  The planning commission may require that applicants provide a form of surety, either through escrow account, bond or otherwise, to cover the cost of removal in the event the city must remove the solar system, of an amount and form determined to be reasonable by the planning commission, but in no event to exceed more than 125 percent of the cost of removal and compliance with the additional requirements set forth herein. Such surety shall not be required for municipally or state-owned solar systems. If required, the applicant shall submit a fully inclusive estimate of the costs associated with removal prepared by a qualified engineer. The amount shall include a mechanism for cost of living adjustment.

(Ord. No. 2010-9, 6-21-10)